IClawMINI Series Specifications Comparison Table

| Specification | IClawMINI | IClawMINI Pro | IClawMINI Ultra |
| --- | --- | --- | --- |
| Positioning | Pure OpenClaw Scheduler + Home Cloud Storage | Local Small Model Inference (≤2B parameters) + Home Cloud Storage | High-Performance Local Large Model (7B-30B parameters) + Home Cloud Storage |
| Core SoC | Rockchip RK3568 | Rockchip RK3576 | AMD Ryzen AI Max+ 395 / Intel Core Ultra |
| CPU Architecture | Quad-core Cortex-A55 @ 2.0GHz | 4×Cortex-A72 @ 2.2GHz + 4×Cortex-A53 @ 1.8GHz | 16-core 32-thread Zen 5 (AMD) or High-performance hybrid architecture (Intel) |
| NPU Computing Power | 1 TOPS (can be disabled) | 6 TOPS @INT8 | AMD XDNA 2: 50 TOPS / Intel Integrated NPU: 10-40 TOPS |
| Memory | 4GB LPDDR4 | 8GB LPDDR4X/LPDDR5 | 32GB/64GB LPDDR5/DDR5 (Up to 128GB unified memory) |
| System Storage | 32GB eMMC 5.1 | 64GB eMMC 5.1 | Optional onboard eMMC or handled by NVMe |
| Storage Solution | Dual M.2 NVMe (PCIe 3.0, 2280) | Dual M.2 NVMe (PCIe 3.0, 2280) | Dual M.2 NVMe (PCIe 4.0, 2280, supports RAID) |
| Network Connectivity | Dual Gigabit Ethernet, Wi-Fi 5, Bluetooth 5.0 | Dual Gigabit Ethernet, Wi-Fi 6, Bluetooth 5.2 | Dual 2.5GbE Ethernet, Wi-Fi 7, Bluetooth 5.4 |
| Video Output | HDMI 2.0 (4K@60fps) | HDMI 2.1 (4K@120fps / 8K@30fps decoding) | HDMI 2.1 / DP 2.0, supports multi-display |
| Interface Layout | Front: 2×USB-C (USB 3.0), 3.5mm audio, power button Rear: 2×USB 3.0, USB OTG, HDMI, GPIO, 12V DC | Front: 2×USB-C (USB 3.0), 3.5mm audio, power button Rear: 2×USB 3.0, USB OTG, HDMI, GPIO, 12V DC Optional MIPI CSI | Front: 2×USB-C (USB4), 3.5mm audio, power button Rear: 2×USB 3.2 Gen2, 2×USB 2.0, HDMI, DP, GPIO (optional), 12V/19V DC |
| GPIO Expansion | 2.54mm header (I2C/SPI/UART) | 2.54mm header (I2C/SPI/UART) | Optional 2.54mm header or direct breakout |
| Power Consumption | 4W idle, 12W full load (excluding drives) | 5W idle, 15-20W full load (excluding drives) | 15W idle, 65W full load (including AI workload) |
| Cooling Design | Fully passive cooling (aviation aluminum chassis) | Aviation aluminum passive + intelligent low-noise fan | Aviation aluminum + heat pipe + active fan (high-performance cooling) |
| Dimensions | 12.7cm × 12.7cm × 4.97cm | 12.7cm × 12.7cm × 4.97cm | 15cm × 15cm × 5cm (may be slightly larger) |
| Chassis Material | Aviation aluminum (6063 aluminum alloy) | Aviation aluminum | Aviation aluminum |
| Operating System | IClawOS Lite (Minimal Buildroot/OpenClaw scheduler) | IClawOS Pro (Pre-installed RKNN toolchain, Ollama/llama.cpp) | Windows / Linux dual system, full AI framework support |
| AI Capability | No local model operation | Smooth operation of 2B parameter models (e.g., Qwen2-1.5B, Phi-2) | Smooth operation of 7B-13B quantized models, can attempt 30B-70B models |
| Target Users | Users needing only cloud storage + basic automation, no AI requirements | Users needing local voice assistant, lightweight AI, moderate budget | Users needing high-performance local large models, pursuing flagship AI experience |

**Notes:**

- The Base version can optionally use an NPU-less model to further reduce costs, but the RK3568 standard configuration includes a 1TOPS NPU.

- The Pro version's RK3576 6TOPS NPU fully meets the real-time inference requirements for 2B models, with actual testing achieving approximately 8-10 tokens per second.

- The Ultra version uses the AMD Ryzen AI Max+ 395 as an example; its 50TOPS NPU and 128GB unified memory can support models up to 70B parameters, with actual performance depending on the quantization level. Intel platforms can be selected based on budget and performance requirements.